MILK TEST CRITERIA

| Test ID | Temperature Range (°C) | Humidity Range (%) | System Status | LED Color Output | Relay Status | Buzzer Status | Meaning for Milk Preservation / Possible Milk Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T01 | ≤ 2°C | 80–90% | Too Cold | Blue | OFF | ON | Milk may begin overcooling/freezing; storage below recommended level. Ice crystal formation disrupts fat globules and proteins, causing grainy texture and separation. |
| T02 | 3°C – 5°C | 80–90% | Safe Range | Cyan | OFF | OFF | Ideal preservation temperature and humidity for fresh milk. Milk remains stable and safe for pooling/processing. |
| T03 | > 5°C | 80–90% | Too Hot | Red | ON | ON | Spoilage risk begins. Warm conditions accelerate bacterial growth, reduce shelf life, and may indicate poor storage/transport. |
| T04 | > 8°C | 80–90% | Critical Heat | Red | ON | ON | High bacterial contamination/spoilage danger. Milk should be isolated before mixing with pooled batch. |
| T05 | Any | < 70% | Low Humidity Warning | Yellow | OFF | OFF | Storage environment too dry; may indicate excessive ventilation or poor storage conditions affecting container integrity and cooling efficiency. |
| T06 | Any | 70–79% | Moderate Humidity | Green | OFF | OFF | Acceptable but below ideal range; monitor closely for fluctuations. |
| T07 | Any | 80–90% | Optimal Humidity | Cyan | OFF | OFF | Ideal humidity for dairy cold-room/storage preservation. Maintains stable cooling conditions. |
| T08 | Any | > 90% | High Humidity Warning | Purple | OFF | ON | Excess moisture may cause condensation, mold growth, and bacterial contamination around storage containers/equipment. |
| T09 | Any | > 95% | Critical Moisture | Red Flash | ON | ON | Severe humidity may indicate refrigeration malfunction or poor ventilation; high contamination risk from environmental bacteria. |
| T10 | Sensor Failure | N/A | Error State | None | OFF | OFF | System cannot monitor milk preservation; manual inspection required immediately. |